Haryana Groundwater AI — Dataset & Data-Source Pack

Hackathon-ready data plan for GIS-enabled groundwater-risk prediction

This document organizes the inputs discussed for the Haryana groundwater project. It combines district-level values available in the Haryana Water Resources Atlas 2025 with the official sources needed to build the remaining GIS/ML layers. It is a starter dataset/schema rather than a claim that every variable is available at the same spatial and temporal resolution.

# 1. Inputs for the AI model

| Input | Recommended field(s) | Unit | Suggested resolution | Primary source |
| --- | --- | --- | --- | --- |
| Rainfall | rainfall_daily; rainfall_30d; rainfall_60d; rainfall_90d; annual_rainfall | mm | Daily / district or station/grid | IMD; Haryana Water Resources Atlas 2025 |
| Rainfall lag | lag_7d; lag_15d; lag_30d; lag_60d; lag_90d | days / mm | Derived | Calculate from rainfall + groundwater-level time series |
| Soil | soil_type; soil_area; soil_health_index; pH/EC where available | class / acres / index | District / GIS polygon | SLUSI/ICAR; Haryana Water Resources Atlas 2025 |
| Geology / hydrogeology | hydrogeology_class; aquifer_code; aquifer_type | class | GIS polygon | CGWB; Haryana Water Resources Atlas 2025 |
| Aquifer type | younger_alluvium; older_alluvium; aeolian_alluvium; sandstone; quartzite etc. | class | GIS polygon | CGWB |
| Groundwater depth | depth_to_water; pre_monsoon; post_monsoon; seasonal_change | m below ground level | Well / district; ideally monthly | CGWB / GWC Haryana |
| Slope | slope | degrees or % | Raster | DEM-derived: SRTM / Copernicus DEM / NRSC-Bhuvan |
| Land use / land cover | LULC class; built_up; agriculture; waterbody; forest | class / area | Raster / polygon | HARSAC / NRSC-Bhuvan |
| Crop | paddy; wheat; cotton; bajra; sugarcane; mustard; other_crop | acres / ha | District / GIS | HARSAC; Haryana Water Resources Atlas 2025 |
| Irrigation | minor_irrigation_units; MI_density; irrigation_intensity | units; units/km²; % | District / GIS | GWC Haryana / HARSAC |
| Extraction | annual_groundwater_extraction; irrigation_extraction; domestic_extraction; industrial_extraction | m³/year or MCM/year | Block / district | CGWB / Haryana Water Resources Atlas 2025 |
| Canals / rivers | canal_length; canal_presence; river_presence; distance_to_canal; distance_to_river | km / binary / km | GIS | Haryana Water Resources Atlas 2025; HARSAC/NRSC |

Important: rainfall lag is not an independently collected variable. It should be engineered from historical rainfall and groundwater-level observations. Slope should also be derived from a DEM rather than treated as a fixed district value.

# 2. Haryana district-level starter dataset

The following fields are transcribed/organized from the Haryana Water Resources Atlas 2025. Recharge values are annual replenishable recharge ranges by aquifer (metres of water equivalent), not a direct measurement of daily recharge. Soil areas are in thousand acres; crop and irrigation areas are in acres.

# 3. District dataset — rainfall, recharge, soil, irrigation and crops

Rainfall columns are annual district rainfall (mm), 2000–2023. Recharge is the atlas's annual replenishable recharge range (m) for the district's listed aquifer context. Crop areas are 2023 Kharif and 2023–24 Rabi.

| District | rain_2015 | rain_2016 | rain_2017 | rain_2018 | rain_2019 | rain_2020 | rain_2021 | rain_2022 | rain_2023 | dominant_soil | recharge_min_m | recharge_max_m | MI_density_units_per_km2 | kharif_paddy_acres | kharif_cotton_acres | rabi_wheat_acres | rabi_mustard_acres |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ambala | 748 | 503 | 810 | 896 | 805 | 1032 | 1032 | 425 | 483 | Old Alluvial Plain | 0.24 | 0.49 | 16.19 | 192855 | 33998 | 182791 | 13355 |
| Bhiwani | 254 | 358 | 275 | 310 | 261 | 406 | 370 | 187 | 152 | Aeolian Plain | 0.10 | 0.20 | 8.51 | 93149 | 378567 | 238332 | 334667 |
| Charkhi Dadri | 254 | 358 | 275 | 310 | 261 | 406 | 513 | 277 | 256 | Fluvioaeolian Plain | 0.00 | 0.00 | 16.34 | 39509 | 155063 | 104672 | 126973 |
| Faridabad | 709 | 688 | 604 | 579 | 429 | 672 | 672 | 250 | 275 | Old Alluvial Plain | 0.31 | 0.35 | 13.85 | 56996 | 0 | 57470 | 8836 |
| Fatehabad | 206 | 201 | 162 | 181 | 156 | 354 | 351 | 358 | 134 | Old Alluvial Plain with Sand Dunes | 0.15 | 0.48 | 14.64 | 336067 | 55838 | 436232 | 65919 |
| Gurugram | 451 | 440 | 280 | 425 | 369 | 584 | 582 | 555 | 334 | Aravalli Piedmont / Old Alluvial | 0.15 | 0.30 | 15.81 | 24309 | 4792 | 72357 | 58243 |
| Hisar | 303 | 317 | 348 | 240 | 236 | 383 | 384 | 425 | 171 | Old Alluvial Plain with Sand Dunes | 0.14 | 0.26 | 17.13 | 260940 | 304829 | 444236 | 292704 |
| Jhajjar | 412 | 449 | 396 | 428 | 202 | 440 | 446 | 357 | 271 | Old Alluvial Plain | 0.27 | 0.46 | 21.90 | 136030 | 12077 | 199469 | 75680 |
| Jind | 443 | 400 | 369 | 448 | 269 | 494 | 492 | 402 | 216 | Old Alluvial Plain with Sand Dunes | 0.22 | 0.52 | 20.54 | 481883 | 55959 | 511940 | 35104 |
| Kaithal | 395 | 294 | 445 | 603 | 340 | 419 | 430 | 409 | 315 | Old Alluvial Plain | 0.20 | 0.34 | 21.12 | 425482 | 0 | 437921 | 6599 |
| Karnal | 624 | 470 | 654 | 866 | 526 | 665 | 662 | 555 | 507 | Old Alluvial Plain | 0.26 | 0.57 | 27.22 | 414101 | 0 | 446869 | 8581 |
| Kurukshetra | 322 | 264 | 451 | 637 | 425 | 614 | 604 | 545 | 681 | Old Alluvial Plain | 0.16 | 0.26 | 24.56 | 189509 | 0 | 229234 | 34552 |
| Mahendragarh | 293 | 401 | 345 | 367 | 265 | 487 | 484 | 399 | 342 | Aravalli Piedmont | 0.09 | 0.16 | 16.41 | 0 | 47656 | 91355 | 236808 |
| Mewat (Nuh) | 546 | 427 | 509 | 561 | 360 | 572 | 571 | 482 | 341 | Aravalli Piedmont / Old Alluvial | 0.16 | 0.26 | 10.69 | 40111 | 5392 | 149672 | 78921 |
| Palwal | 398 | 430 | 304 | 383 | 304 | 481 | 512 | 394 | 267 | Recent Flood Plain / Aravalli Hills | 0.28 | 0.46 | 48.80 | 87197 | 21864 | 217218 | 19374 |
| Panchkula | 651 | 445 | 469 | 342 | 473 | 486 | 727 | 782 | 864 | Siwalik Hills / Piedmont | 0.33 | 0.41 | 4.44 | 35069 | 0 | 30099 | 3650 |
| Panipat | 498 | 322 | 371 | 437 | 255 | 567 | 557 | 377 | 352 | Recent Flood Plain | 0.24 | 0.32 | 27.98 | 192940 | 0 | 180702 | 8186 |
| Rewari | 383 | 484 | 342 | 426 | 438 | 491 | 499 | 341 | 331 | Fluvioaeolian Plain | 0.13 | 0.31 | 8.75 | 9108 | 18392 | 96483 | 171188 |
| Rohtak | 321 | 310 | 296 | 368 | 242 | 608 | 602 | 436 | 332 | Old Alluvial Plain with Sand Dunes | 0.29 | 0.62 | 30.37 | 186576 | 20134 | 227708 | 26182 |
| Sirsa | 300 | 177 | 172 | 267 | 270 | 277 | 283 | 219 | 177 | Aeofluvial Plain | 0.14 | 0.31 | 16.37 | 285444 | 0 | 552291 | 266695 |
| Sonipat | 453 | 287 | 500 | 431 | 339 | 629 | 645 | 291 | 323 | Old Alluvial Plain | 0.24 | 0.59 | 15.57 | 276495 | 11077 | 292029 | 45034 |
| Yamunanagar | 771 | 877 | 948 | 1066 | 824 | 1005 | 988 | 988 | 1178 | Active Flood Plain / Piedmont | 0.30 | 0.37 | 18.23 | 159918 | 0 | 203970 | 23767 |

# 4. Soil dataset — district × soil-type area

| District | TGA ('000 acres) | Active Floodplain | Aeofluvial | Aeolian | Aravalli Hills | Aravalli Piedmont | Fluvioaeolian | Old Alluvial | Old Alluvial + Sand Dunes | Piedmont | Recent Floodplain | Siwalik |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ambala | 388.94 | 74.81 | 0 | 0 | 0 | 0 | 0 | 92.10 | 0 | 51.55 | 154.58 | 15.90 |
| Bhiwani | 848.06 | 0 | 0 | 285.80 | 2.68 | 0 | 80.08 | 0 | 479.51 | 0 | 0 | 0 |
| Charkhi Dadri | 332.60 | 0 | 12.05 | 2.33 | 27.08 | 0 | 255.32 | 0 | 35.82 | 0 | 0 | 0 |
| Faridabad | 183.10 | 14.95 | 0 | 0 | 9.44 | 54.85 | 0 | 62.38 | 11.09 | 0 | 30.39 | 0 |
| Fatehabad | 627.15 | 13.60 | 155.57 | 0 | 0 | 0 | 0 | 5.14 | 305.02 | 0 | 147.83 | 0 |
| Gurugram | 310.86 | 0 | 0 | 0 | 16.91 | 108.07 | 55.85 | 20.24 | 62.77 | 0 | 47.04 | 0 |
| Hisar | 984.22 | 0 | 19.79 | 7.82 | 0 | 0 | 0 | 216.86 | 739.74 | 0 | 0 | 0 |
| Jhajjar | 453.19 | 0 | 0 | 0 | 1.41 | 0 | 0 | 300.95 | 94.95 | 0 | 55.87 | 0 |
| Jind | 667.68 | 0 | 0 | 0 | 0 | 0 | 0 | 530.95 | 136.72 | 0 | 0 | 0 |
| Kaithal | 572.54 | 12.86 | 0 | 0 | 0 | 0 | 0 | 518.04 | 0 | 0 | 41.65 | 0 |
| Karnal | 622.70 | 60.29 | 0 | 0 | 0 | 0 | 0 | 333.30 | 6.63 | 0 | 222.48 | 0 |
| Kurukshetra | 378.07 | 0 | 0 | 0 | 0 | 0 | 0 | 378.07 | 0 | 0 | 0 | 0 |
| Mahendragarh | 469.25 | 0 | 0 | 19.92 | 0 | 248.76 | 197.48 | 0 | 3.10 | 0 | 0 | 0 |
| Mewat (Nuh) | 372.39 | 6.75 | 0 | 0 | 18.69 | 237.27 | 0 | 109.68 | 0 | 0 | 0 | 0 |
| Palwal | 335.82 | 42.36 | 0 | 0 | 116.81 | 0 | 0 | 0 | 0 | 0 | 176.64 | 0 |
| Panchkula | 221.90 | 4.06 | 0 | 0 | 0 | 0 | 0 | 22.72 | 0 | 17.06 | 39.22 | 138.84 |
| Panipat | 313.33 | 0 | 0 | 0 | 0 | 0 | 0 | 73.11 | 0 | 0 | 240.22 | 0 |
| Rewari | 393.89 | 0 | 0 | 0 | 40.03 | 0 | 353.86 | 0 | 0 | 0 | 0 | 0 |
| Rohtak | 431.20 | 0 | 0 | 0 | 0 | 0 | 0 | 290.33 | 140.86 | 0 | 0 | 0 |
| Sirsa | 1056.87 | 0 | 793.87 | 3.04 | 0 | 0 | 0 | 5.14 | 121.44 | 0 | 133.38 | 0 |
| Sonipat | 524.36 | 0 | 0 | 0 | 0 | 0 | 0 | 316.75 | 159.98 | 0 | 47.62 | 0 |
| Yamunanagar | 436.88 | 153.38 | 0 | 0 | 0 | 0 | 0 | 7.54 | 0 | 210.65 | 35.44 | 29.87 |

# 5. Hydrogeology / aquifer dataset

| Code | Aquifer / hydrogeology | Area | % Haryana |
| --- | --- | --- | --- |
| AL01 | Younger Alluvium | 701,778 acres | 6.40% |
| AL02 | Boulder/Gravels | 116,881 acres | 1.10% |
| AL03 | Older Alluvium | 6,600,669 acres | 60.40% |
| AL04 | Aeolian Alluvium | 3,262,527 acres | 29.90% |
| ST01 | Sandstone/Conglomerates | 104,773 acres | 1.00% |
| ST02 | Sandstone/Clay | 48,680 acres | 0.45% |
| GR01 | Granite / acidic rocks | 988 acres | 0.01% |
| QZ01 | Quartzites | 88,711 acres | 0.80% |

The atlas states that older alluvium, aeolian deposits and younger alluvium together cover about 97% of Haryana, and that favorable recharge conditions occur over about 64% of the state's area.

# 6. Groundwater-level / response dataset

Use CGWB/GWC well observations as the target variable. For ML, keep each observation with date, latitude, longitude, well ID, depth to water (m bgl), and measurement season. Then join rainfall features using the same location and time window.

| Field | Example / format | Use |
| --- | --- | --- |
| well_id | unique string | Panel/time-series key |
| date | YYYY-MM-DD | Time axis |
| lat, lon | decimal degrees | GIS location |
| depth_to_water_m | numeric | Target / predictor |
| season | pre-monsoon / monsoon / post-monsoon / winter | Seasonality |
| rain_7d_mm | derived | Short lag |
| rain_30d_mm | derived | Recharge lag |
| rain_60d_mm | derived | Recharge lag |
| rain_90d_mm | derived | Recharge lag |
| water_level_change_m | derived | Target: rise/fall |

# 7. Important groundwater-stress data already available

The atlas's October 2013–October 2023 decadal water-level fluctuation is categorized by area. Negative values mean decline; positive values mean rise.

| District | TGA acres | < -4m | -4 to -2m | -2 to 0m | 0 to 2m | 2 to 4m | >4m |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Hisar | 984219.22 | 7798.88 | 77598.90 | 235136.37 | 423869.37 | 209010.10 | 30805.59 |
| Kaithal | 572542.29 | 538959.70 | 27439.44 | 4095.44 | 819.09 | 409.54 | 819.09 |
| Karnal | 622704.60 | 310534.03 | 58506.41 | 91237.27 | 134605.66 | 27821.23 | 0 |
| Kurukshetra | 378070.65 | 340734.81 | 16674.26 | 8699.61 | 8699.61 | 2899.87 | 362.48 |
| Sirsa | 1056868.09 | 494210.10 | 369129.72 | 116116.96 | 56632.48 | 11407.98 | 9370.84 |
| Haryana | 10925006.26 | 4184355.95 | 1682794.54 | 2722302.20 | 1729401.41 | 453037.80 | 153114.40 |

# 8. Official datasets / download sources for the remaining GIS layers

| Source | Best use | Official link |
| --- | --- | --- |
| Haryana Water Resources Atlas 2025 | Rainfall, soil, hydrogeology/aquifers, groundwater levels, recharge, tubewells, canals, irrigation, crops, LULC, water quality | https://hwra.org.in/Atlas2025/HWRAAtlas2025.pdf |
| CGWB Data Download | Groundwater datasets and monitoring data | https://cgwb.gov.in/en/data-download |
| CGWB Haryana | District groundwater reports for Haryana districts | https://cgwb.gov.in/en/node/1759 |
| CGWB Groundwater Level Monitoring | Historical well-level datasets; four measurements/year and long time series | https://cgwb.gov.in/hi/ground-water-level-monitoring |
| India Meteorological Department (IMD) | District/station rainfall; daily, weekly, monthly and cumulative rainfall | https://mausam.imd.gov.in/imd_latest/contents/rainfallinformation.php |
| Haryana Water Resources Authority | Groundwater categorization, water action plans, Haryana resources | https://hwra.org.in/about.html |

# 9. What is still missing / should be derived

Rainfall lag: derive 7/15/30/60/90-day cumulative rainfall and identify the best lag statistically for each hydrogeological zone.

Slope: download a DEM and calculate slope in GIS; keep mean/max slope per grid or village.

Land-use change: obtain multi-year LULC (e.g., 2015–2025) so the model can detect increasing built-up area and decreasing infiltration.

Extraction: use block/district groundwater-extraction estimates and, if possible, well/pump-level observations.

Canal/river proximity: convert canal/river layers into distance-to-canal and distance-to-river rasters.

Water quality: add EC/TDS, nitrate, fluoride, arsenic, salinity where available; these are important for the 'quality risk' component.

Beneficiaries/schemes: create a separate scheme-beneficiary table with village, scheme, year, beneficiary count, intervention type and coordinates.

# 10. Recommended ML master table

For the hackathon, use one row per spatial unit per time step. A practical MVP is district-month or block-month; a stronger version is grid/well-month.

date

district/block/grid_id

lat

lon

rainfall_mm

rain_7d_mm

rain_15d_mm

rain_30d_mm

rain_60d_mm

rain_90d_mm

soil_type

soil_pH

soil_EC

soil_health_index

hydrogeology

aquifer_type

depth_to_water_m

depth_change_m

slope_deg

lulc_class

paddy_area

wheat_area

cotton_area

bajra_area

mustard_area

irrigation_density

irrigation_intensity

groundwater_extraction_MCM

canal_distance_km

river_distance_km

recharge_estimate_MCM

groundwater_risk_score

# 11. Recharge calculation to use with the dataset

For a rainfall-infiltration estimate, a simplified form is:

Recharge volume ≈ Rainfall × Infiltration factor × Area

For a water-level-fluctuation estimate, use ΔS = h × Sy × A, where h is the groundwater-level rise, Sy is specific yield, and A is the aquifer area. Do not treat ΔS as rainfall recharge without accounting for other recharge/discharge components such as canal seepage, irrigation return flow and extraction.

# 12. Source notes

The Haryana Water Resources Atlas 2025 explicitly provides district-wise tables for soil, hydrogeology, LULC, rainfall, groundwater-level fluctuations, annual replenishable recharge, tubewell depth, irrigation, cropping pattern, canal networks and groundwater utilization. It also states that rainfall data were processed by HARSAC into 5×5 km gridded rainfall for the atlas.

The numerical tables in this document are intended as a clean starter dataset for the hackathon. Before model training, download the source datasets and preserve their original metadata, spatial resolution, dates, units and source organization.